Importing virtual machine image

This process will import the .ova image into the virtualbox installation.

- Open virtualbox

- Got to file > import appliance

- Open the .ova image to import

- Set machine base folder to C:\cpre488\mp-4

- Leave all default settings for appliance settings

- Click import at the bottom of the window

Virtual machine installation location

The virtual machine instance will be installed at C:\cpre488\mp-4, this is a local folder that all students will be able to access with read-write permissions. This way multiple students will share the same virtual machine instance

Exporting virtual machine image

Once the VM image has been setup need to export it to a .ova file to be installed

- Shutdown VM

- File > export appliance

- Open Virtualization Format 2.0

- Include only NAT network adapter MAC address

- Write manifest file

- Do not include ISO image files

- Export

Install guest additions

Install additional drivers to the guest machine to improve performance

- VM must have an optical drive for this to work:

  - Shutdown the VM

  - Machine settings > storage > SATA controller > add optical drive

  - Leave empty button at bottom of window

  - Start the VM

- Devices > insert guest additions CD

- Mount CD

- In CD directory run sudo ./VBoxLinuxAdditions.run

- Restart VM after installation

### Adding the Virtual Machine Instance

New students will not be able to see the VM instance until it is imported.

- Open Virtual box application

- Go to machine -> add

- Navigate to the VM installation, C:\cpre488\mp-4\

- Open the .vbox file

- The virtual machine should now be available

### Folder Sharing with the Virtual Machine

- Go to VM settings, machine -> settings then shared folders

- On the far right click the blue folder with a green cross to add a new shared folder

- Set the folder path to a folder on the windows system to share with the VM

- Select Auto-mount

- For the mount point enter ‘/home/bitcraze/shared-folder’

- Select make permanent

- Select ok on both dialog boxes to complete the shared folder setup

- You should now be able to navigate to /home/bitcraze/shared-folder in the VM and see the files in your shared folder

How to make a VM drive immutable:

- Install VM image as usual, setting files and installations that will be reset to on reboot

- After installed, shutdown the vm

- Option 1: Using the GUI (don't need admin privileges)

  - Open File > Virtual Media Manager

  - Select the .vdi file for the VM instance

  - Select Properties at the top of the window

  - Under attributes tab, change type from normal to immutable

  - Click apply at the bottom of the window

  - Virtual box will warn you the drive must be detached from the vm instance, click release

  - Close the virtual media manager window

  - The storage drive should now show up as immutable

- Option 2: Using the command line (need admin privileges to open cmd in c drive)

  - Go into VM storage settings

  - Note the host OS location of the .vdi file, this will be used later

  - Right click the .vdi drive selecting “Remove attachment”

  - Open command prompt, navigate to the virtualbox installation folder

    - By default C:\Program Files\Oracle\VirtualBox\

  - Run the following command to set the .vdi hard drive to be immutable

    - vboxmanage.exe modifyhd <path of the .vdi file found earlier> --type immutable

      - Use --type normal to revert to a normal hard drive

  - After successful execution, reattach the .vdi drive under VM storage settings

    - SATA controller, Add hard disk

    - The .vdi file should show up in the list of hard drives

  - Click ok

  - Should now show the storage drive as immutable

### Exporting Work From the Virtual Machine

With a bare git repository on the host machine

- Create a bare repository on your removable media or your x drive

  - ‘git init --bare my_repo_name.git’

  - This will create a folder called my_repo_name.git with no working tree.

- Share this folder with the VM and mount it at ‘/home/bitcraze/shared-folder/‘, see details on sharing a folder with the VM

- Once the bare repository is accessible from within the VM

- The Microcart repository should already be setup to use ‘/home/bitcraze/shared-folder/’ as a remote, check this with ‘git remote -v’

- Commit and push your changes to your shared folder

  - If this fails try setting the remote url again with ‘git remote set-url origin <absolute path to shared folder>’

- Your changes will now be on the bare repo in your shared folder

With normal GitHub/GitLab repository

- Note: the first commit will take a while as all the git history must be uploaded

- Create a new blank repo on github or gitlab, obtain the url or ssh address to the repo

- In the Microcart folder run ‘git remote set-url origin <address of remote repo>’

  - Note this set-url command must be run each time the VM is rebooted due to the immutable hard drive setup

- Commit and push your changes as normal

### Importing Work Into the Virtual Machine

With a bare git repository on the host machine

- Ensure the bare repository is accessible within the VM, see details on sharing a folder with the VM

- The Microcart repo should already be setup to use ‘/home/bitcraze/shared-folder/’ as a remote

- From within the Microcart folder run ‘git pull’

With normal GitHub/GitLab repository

- Set the remote repository with ‘git remote set-url origin <address of remote repo>’

- From within the Microcart folder run ‘git pull’

### Final export from the virtual machine

Final export will export all files that have been modified since the original state of the Microcart repo.

- Ensure folder sharing and your git repository are setup correctly

- Commit all changes within the git repository to be exported

- From the root of the Microcart folder, Run ‘cp -pv --parents $(git diff Lab_Part_2_tag --name-only) DESTINATION-DIRECTORY’

  - The destination should be a shared folder so the files are accessible from the host machine

  - This will copy all modified files since the Lab_Part_2_tag into the destination directory